# Custom 32‑Bit CPU Simulator Platform

Comprehensive Project Documentation, Architecture Explanation, and Development Report

# 1. Introduction

This project implements a complete educational CPU simulator platform that allows users to write assembly instructions,

translate them into binary, simulate execution, visualize registers and memory, and analyze program behavior through

a web‑based interface.

The platform was designed to help understand how processors work internally, including instruction encoding,

register manipulation, stack operations, memory architecture, and execution pipelines.

The system combines a Python backend CPU simulation engine with a modern React frontend dashboard.

# 2. Motivation

Understanding CPU architecture normally requires hardware simulators or low‑level development environments.

Many students find it difficult to visualize how assembly instructions translate into machine operations.

This project solves that problem by building an interactive web‑based simulator where users can observe:

• Register updates

• Memory modifications

• Binary instruction generation

• Stack operations

• Pipeline execution

• VM to assembly translation

This makes computer architecture concepts easier to learn.

# 3. System Overview

The system consists of two major subsystems.

Backend (Python FastAPI)

Responsible for CPU simulation, instruction execution, binary generation, VM translation, and analysis.

Frontend (React + Vite + Tailwind)

Responsible for visualization of CPU state including registers, memory, stack, pipeline and instruction flow.

The frontend communicates with the backend using REST APIs.

# 4. Technology Stack

Backend Technologies:

Python

FastAPI

Custom CPU simulation engine

Frontend Technologies:

React

Vite

Tailwind CSS

Axios

Development Tools:

VS Code

Logisim Evolution (for digital logic experimentation)

# 5. Custom CPU Architecture

The CPU implemented in this project is a custom architecture designed specifically for educational simulation.

Key features:

• 16 general purpose registers

• Simulated memory (4096 locations)

• Stack based operations

• Instruction encoding

• Binary machine code generation

• Fetch‑Decode‑Execute cycle simulation

The architecture supports arithmetic, memory operations, and stack manipulation.

# 6. Registers

The CPU contains 16 registers named R0 to R15.

Registers store intermediate values used during computation.

Register operations are significantly faster than memory operations.

Examples of register instructions:

LOADI R1 10

ADD R1 R2

SUB R3 R4

# 7. Memory Architecture

The simulator contains a RAM structure implemented as an array with 4096 memory locations.

Memory layout conceptually contains:

Code Segment – instructions loaded from the program editor

Data Segment – variable storage

Heap – dynamic allocation (future expansion)

Stack – push and pop operations

Memory instructions include:

LOAD

STORE

PUSH

POP

# 8. Stack Architecture

The stack is used for temporary data storage and function style operations.

The stack pointer (SP) tracks the top element of the stack.

Example stack operations:

PUSH R1

POP R2

These operations simulate how real processors manage stack memory.

# 9. Instruction Set Architecture

The CPU supports a custom instruction set including:

LOADI register value

LOAD register address

STORE register address

ADD register register

SUB register register

MUL register register

DIV register register

PUSH register

POP register

HALT

These instructions were designed specifically for this CPU architecture.

# 10. Binary Encoding

Assembly instructions are translated into binary machine instructions.

Example:

Assembly:

LOADI 1 10

Binary representation:

0001000100001010

Binary fields include opcode, register index and immediate value.

This binary representation is displayed in the Binary Viewer component.

# 11. Fetch‑Decode‑Execute Cycle

Every instruction follows a three stage execution cycle.

Fetch:

The CPU fetches the instruction from memory using the program counter.

Decode:

The instruction opcode is decoded to determine the operation.

Execute:

The CPU performs the required operation such as arithmetic or memory access.

After execution, the program counter moves to the next instruction.

# 12. Backend Architecture

Backend components include:

main.py – FastAPI server entry point

cpu.py – CPU execution engine

memory.py – memory simulation

assembler – assembly to binary translation

vm_translator – VM language conversion

analyzer – code analysis and performance metrics

modes.py – user and developer mode control

# 13. Assembler System

The assembler converts assembly language into machine binary instructions.

The assembler module consists of:

parser.py – parses assembly instructions

encoder.py – converts parsed instructions into binary

assembler.py – manages full assembly process

This allows assembly programs written by the user to be executed by the CPU.

# 14. VM Translator

The VM translator converts high level VM instructions into assembly instructions.

Example:

VM:

push constant 7

push constant 8

add

Translated assembly:

LOADI R0 7

PUSH R0

LOADI R0 8

PUSH R0

POP R1

POP R2

ADD R1 R2

PUSH R1

# 15. Code Analyzer

The analyzer module evaluates assembly code quality.

It detects inefficient code patterns and provides suggestions to the developer.

Example suggestions:

Use registers instead of repeated memory loads.

Reduce unnecessary instructions.

# 16. Performance Analyzer

This module measures runtime statistics including:

Instruction count

Memory accesses

Execution cycles

These metrics help developers understand program efficiency.

# 17. Frontend Architecture

The frontend dashboard provides visual representations of CPU state.

Key components include:

Editor.jsx – assembly code editor

Registers.jsx – register visualization

Memory.jsx – memory grid

StackView.jsx – stack display

BinaryViewer.jsx – machine code display

Pipeline.jsx – pipeline stage display

FlowGraph.jsx – instruction flow visualization

ISAReference.jsx – instruction documentation

Guidelines.jsx – dynamic code suggestions

# 18. Binary Viewer

BinaryViewer.jsx displays machine code generated by the assembler.

Each instruction is shown in binary format with its instruction index.

# 19. Developer Mode vs User Mode

The platform includes two operating modes.

User Mode:

Simplified interface for running programs.

Developer Mode:

Advanced debugging including binary view, pipeline visualization,

memory monitoring and VM translation tools.

# 20. Instruction Workflow

The instruction execution workflow is:

User writes assembly code

Frontend sends program to backend

Assembler converts code into binary

CPU loads instructions

User runs or steps through program

Registers and memory update

Frontend dashboard visualizes the state

# 21. Why This CPU Is Custom

This CPU is considered custom because the architecture, instruction set,

binary encoding, and execution model were designed specifically for this project.

Unlike standardized architectures such as x86 or ARM, this CPU defines its own

instruction formats and execution behavior.

# 22. Comparison With Hack CPU

Hack CPU from the Nand2Tetris course uses a fixed 16‑bit architecture.

Differences:

Hack CPU:

Simple hardware simulator

Fixed instruction format

Minimal debugging tools

Custom CPU Simulator:

Web based platform

Extended visualization tools

Stack architecture

Binary viewer

Pipeline visualization

VM translator

# 23. Future Improvements

Possible future enhancements include:

Branching instructions

Function call instructions

Cache simulation

Multi‑core CPU simulation

Interrupt handling

Compiler support

Instruction optimization analysis

Graphical pipeline animation

# 24. Logisim Evolution Contribution

Logisim Evolution can be used to design the digital logic corresponding

to the simulated CPU architecture.

Potential Logisim modules include:

ALU design

Register file

Program counter

Control unit

Instruction decoder

Memory module

# 25. Team Work Division (4 Members)

Member 1:

Backend CPU architecture implementation

cpu.py, memory.py, instruction execution

Member 2:

Assembler and VM translator development

parser.py, encoder.py, assembler.py, vm_translator.py

Member 3:

Frontend UI development

React dashboard components and visualization

Member 4:

Digital hardware simulation and documentation

Logisim Evolution CPU design and project report preparation

Architecture Level Difference

Feature

Nand2Tetris

Your CPU

CPU Type

Hack CPU

Custom CPU

Instruction Size

16-bit

Custom (your encoding)

Registers

A, D only

16 registers (R0–R15)

Stack

VM managed

CPU + VM stack

Execution

Hardware simulation

Python CPU simulation

Interface

Java GUI

Web dashboard

So the biggest difference:

👉 Nand2Tetris CPU has only 2 registers

👉 Your CPU has 16 registers

That already makes your ISA very different.

2️⃣ Assembly Language Difference

Nand2Tetris Assembly

Example:

Copy code

@10

D=A

@0

M=D

Meaning:

Copy code

D = 10

RAM[0] = D

Instructions are divided into:

A-instruction

Copy code

@value

Example:

Copy code

@15

C-instruction

Copy code

dest=comp;jump

Example:

Copy code

D=M

D=D+A

0;JMP

Your Assembly

Example:

Copy code

LOADI 1 10

LOADI 2 5

ADD 1 2

STORE 1 200

HALT

Your syntax is instruction + operands.

Key Syntax Difference

Feature

Nand2Tetris

Your CPU

Instruction format

symbolic

opcode style

Registers

A,D only

R0–R15

Memory

M symbol

address number

Operations

comp based

explicit instructions

3️⃣ Instruction Style Difference

Nand2Tetris

Instructions are expression based.

Example:

Copy code

D=D+A

This means:

Copy code

D = D + A

But your CPU uses explicit instructions.

Example:

Copy code

ADD 1 2

Meaning:

Copy code

R1 = R1 + R2

So your ISA is closer to RISC architecture.

4️⃣ Binary Encoding Difference

Nand2Tetris Binary

Instruction size:

Copy code

16 bits

Example:

Copy code

1110110000010000

Structure:

Copy code

111 a c1 c2 c3 c4 c5 c6 d1 d2 d3 j1 j2 j3

Very complex.

Your Binary

Example:

Copy code

0001000100001010

Structure (example):

Copy code

opcode | register | value

Much simpler encoding.

5️⃣ VM Language Difference

Nand2Tetris VM

Example:

Copy code

push constant 7

push constant 8

add

pop local 0

VM supports many segments:

Copy code

local

argument

this

that

pointer

temp

static

Your VM

Your VM currently supports simpler operations:

Copy code

push

pop

add

sub

mul

div

And translates them into your custom assembly.

6️⃣ Execution Model Difference

Nand2Tetris

Execution path:

Copy code

VM

↓

Assembly

↓

Hack CPU Hardware

Runs in hardware simulator.

Your Project

Execution path:

Copy code

VM

↓

Assembly

↓

Binary

↓

Python CPU Simulator

Runs in software CPU engine.

7️⃣ Visualization Difference

Nand2Tetris

Shows:

registers

RAM

screen

keyboard

Very basic.

Your Simulator

Your platform shows:

registers

RAM

stack

binary code

pipeline

flow graph

VM translation

guidelines

developer mode

So your system is more visualization-focused.

8️⃣ Stack Implementation Difference

Nand2Tetris

Stack pointer stored at:

Copy code

RAM[0]

Stack starts at:

Copy code

256

Your CPU

Stack pointer is inside CPU:

Copy code

sp variable

Stack memory region is controlled by CPU.

9️⃣ Programming Model Difference

Nand2Tetris

Two registers:

Copy code

A

D

Memory operations:

Copy code

M

Your CPU

Registers:

Copy code

R0–R15

More flexible.

🔟 Your CPU is Actually Closer to RISC

Your ISA looks more like:

Copy code

MIPS

RISC-V

Because instructions look like:

Copy code

ADD R1 R2

LOAD R1 100

STORE R1 200

This is RISC-style.

1️⃣1️⃣ How to Explain in Viva

You can say this:

Our CPU simulator was inspired by the Nand2Tetris architecture, but we designed a custom instruction set architecture with 16 registers, explicit instruction syntax, and a web-based execution environment. Unlike the Hack CPU which uses only A and D registers and symbolic instructions, our CPU supports multiple registers, stack operations, binary visualization, and pipeline analysis.

1️⃣2️⃣ Biggest Differences (Simple Summary)

Feature

Nand2Tetris

Your CPU

Registers

2

16

Assembly style

symbolic

opcode based

Execution

hardware sim

software CPU

Visualization

basic

advanced

Stack

VM controlled

CPU + VM

Interface

Java

Web UI

✅ Your project is actually a more advanced visualization platform than Nand2Tetris.